LLM CLUSTER BUILD CHECKLIST

Kubernetes-First · k3s + llama.cpp · 2 Nodes → Scale

Lenovo P1 Gen3  ·  64 GB RAM  ·  T1000 4 GB  ·  i7 vPro 10th Gen

Precise · Fast · Tunable · Fully On-Premise  |  2026

How to Use This Checklist

Work through each phase in order. Tick □ when done. Every step has the exact command. Do not skip steps … each is a dependency for the next.

Hardware: Lenovo P1 Gen3  ·  Intel i7 vPro 10th Gen  ·  64 GB DDR4  ·  NVIDIA Quadro T1000 4 GB VRAM Max-Q  ·  Ubuntu 22.04 LTS

Start: 2 nodes (node-00 with screen = control plane, node-01 headless = worker). To add more nodes: repeat Phase C+D+E+F for each new node, then run step 90.

PHASE A — Hardware & Physical Setup

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 1 | Connect both nodes to switch with Cat6 cables | node-00 and node-01 → switch → router/internet |
| □ | 2 | Disable Secure Boot in BIOS on both nodes | Required for NVIDIA proprietary driver installation |
| □ | 3 | Set boot order: USB first, then NVMe | Needed for Ubuntu install |
| □ | 4 | Flash 2x USB drives with Ubuntu Server 22.04 LTS ISO | balenaEtcher or Rufus · ubuntu.com/download/server |

PHASE B — OS Installation (both nodes)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 5 | Install Ubuntu Server on node-00 | hostname: node-00 · IP: 192.168.10.10/24 · GW: 192.168.10.1 · SSH: yes · no GUI |
| □ | 6 | Install Ubuntu Server on node-01 | hostname: node-01 · IP: 192.168.10.11/24 · GW: 192.168.10.1 · SSH: yes · no GUI |
| □ | 7 | Update packages on both nodes | sudo apt update && sudo apt upgrade -y |
| □ | 8 | Add /etc/hosts entries on node-00 | echo '192.168.10.11 node-01' ¦ sudo tee -a /etc/hosts |
| □ | 9 | Add /etc/hosts entries on node-01 | echo '192.168.10.10 node-00' ¦ sudo tee -a /etc/hosts |
| □ | 10 | Generate SSH key on node-00 | ssh-keygen -t ed25519 -f ~/.ssh/id_ed25519 -N '' |
| □ | 11 | Copy SSH public key to node-01 | ssh-copy-id ubuntu@node-01 |
| □ | 12 | Test passwordless SSH from node-00 to node-01 | ssh node-01 hostname # should print: node-01 |
| □ | 13 | Ping test between nodes | ping -c 3 node-01 (from node-00) |

PHASE C — GPU Drivers & CUDA (both nodes)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 14 | Add NVIDIA CUDA apt keyring on both nodes | wget https://developer.download.nvidia.com/compute/cuda/repos/ubuntu2204/x86_64/cuda-keyring_1.1-1_all.deb && sudo dpkg -i cuda-keyring_1.1-1_all.deb |
| □ | 15 | Update apt | sudo apt update |
| □ | 16 | Install NVIDIA driver 535 | sudo apt install -y nvidia-driver-535 |
| □ | 17 | Install CUDA 12.3 toolkit | sudo apt install -y cuda-toolkit-12-3 (doesn’t work) apt list –installed ¦ grep cuda sudo apt install cuda-compiler-12-3 cuda-libraries-12-3 cuda-libraries-dev-12-3 |
| □ | 18 | Install nvidia-container-toolkit | sudo apt install -y nvidia-container-toolkit |
| □ | 19 | Reboot both nodes | sudo reboot |
| □ | 20 | Validate GPU on node-00 | nvidia-smi # expect: Quadro T1000, 4096 MiB, Driver 535.x |
| □ | 21 | Validate GPU on node-01 | ssh node-01 nvidia-smi # same expected output |
| □ | 22 | Check CUDA version | nvcc --version # expect: release 12.3 |

PHASE D — Build llama.cpp with CUDA (both nodes)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 23 | Install build dependencies on both nodes | sudo apt install -y build-essential cmake git |
| □ | 24 | Clone llama.cpp on both nodes | git clone https://github.com/ggerganov/llama.cpp /opt/llama.cpp |
| □ | 25 | Configure cmake with CUDA enabled | cd /opt/llama.cpp sudo cmake -B build-cuda -DGGML_CUDA=ON -DCMAKE_CUDA_ARCHITECTURES=”75” |
| □ | 26 | Compile llama.cpp (uses all CPU cores, takes Time ~5-10 min) | Sudo cmake --build build-cuda --config Release -j$(nproc) |
| □ | 27 | Verify build succeeded | ./build/bin/llama-server --version |
| □ | 28 | Create models directory on both nodes | mkdir -p /opt/llama.cpp/models |

PHASE E — Download Model & Test Inference

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 29 | Install huggingface_hub on node-00 | pip install huggingface_hub sudo apt install python3-huggingface-hub |
| □ | 30 | Download Llama 3.1 8B Instruct Q4_K_M GGUF (~4.7 GB) | huggingface-cli download bartowski/Meta-Llama-3.1-8B-Instruct-GGUF Meta-Llama-3.1-8B-Instruct-Q4_K_M.gguf --local-dir /opt/llama.cpp/models/ (doesn’t work) sudo hf download bartowski/Meta-Llama-3-8B-Instruct-GGUF Meta-Llama-3-8B-Instruct-Q4_K_M.gguf –-local-dir /opt/llama.cpp/models/ |
| □ | 31 | Run quick CLI inference test on node-00 | cd /opt/llama.cpp && ./build/bin/llama-cli -m models/Meta-Llama-3.1-8B-Instruct-Q4_K_M.gguf -ngl 35 -c 2048 -p 'What is 2+2?' -n 64 |
| □ | 32 | Confirm output shows 'offloaded 35/35 layers to GPU' | Look for this line in the llama-cli output |
| □ | 33 | Check GPU memory usage during inference | nvidia-smi # VRAM used should be ~4.0-4.5 GB |
| □ | 34 | Copy model to node-01 via rsync | rsync -avz --progress /opt/llama.cpp/models/ ubuntu@node-01:/opt/llama.cpp/models/ |
| □ | 35 | Test inference on node-01 | ssh node-01 'cd /opt/llama.cpp && ./build/bin/llama-cli -m models/Meta-Llama-3.1-8B-Instruct-Q4_K_M.gguf -ngl 35 -p "Hello" -n 32' |

PHASE F — k3s Kubernetes Cluster

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 36 | Install k3s server on node-00 (control plane + worker, traefik disabled) | curl -sfL https://get.k3s.io ¦ sh -s - --write-kubeconfig-mode 644 --disable traefik |
| □ | 37 | Verify k3s is running on node-00 | sudo systemctl status k3s # should show: active (running) |
| □ | 38 | Get k3s node token from node-00 | sudo cat /var/lib/rancher/k3s/server/node-token # copy this value |
| □ | 39 | Install k3s agent on node-01 (replace TOKEN) | curl -sfL https://get.k3s.io ¦ K3S_URL=https://192.168.10.10:6443 K3S_TOKEN=<TOKEN> sh - |
| □ | 40 | Verify both nodes are Ready | kubectl get nodes -o wide # both STATUS=Ready within 60s |
| □ | 41 | Configure nvidia-container-toolkit for k3s containerd on node-00 | sudo nvidia-ctk runtime configure --runtime=containerd && sudo systemctl restart k3s |
| □ | 42 | Configure nvidia-container-toolkit for k3s containerd on node-01 | ssh node-01 'sudo nvidia-ctk runtime configure --runtime=containerd && sudo systemctl restart k3s-agent' |
| □ | 43 | Deploy NVIDIA Device Plugin DaemonSet | kubectl apply -f https://raw.githubusercontent.com/NVIDIA/k8s-device-plugin/v0.14.0/nvidia-device-plugin.yml |
| □ | 44 | Wait for NVIDIA Device Plugin pods to be Running | kubectl get pods -n kube-system -l name=nvidia-device-plugin-ds # both Running |
| □ | 45 | Verify GPU resource visible on each node | kubectl get nodes -o json ¦ grep -A4 'nvidia.com/gpu' # expect: "nvidia.com/gpu": "1" per node |
| □ | 46 | Run GPU test pod to validate scheduling | kubectl run gpu-test --image=nvidia/cuda:12.3.0-base-ubuntu22.04 --restart=Never --limits='nvidia.com/gpu=1' -- nvidia-smi |
| □ | 47 | Check GPU test pod logs | kubectl logs gpu-test # expect: T1000 4096 MiB |
| □ | 48 | Delete GPU test pod | kubectl delete pod gpu-test |

PHASE G — MetalLB Bare-Metal Load Balancer

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 49 | Deploy MetalLB | kubectl apply -f https://raw.githubusercontent.com/metallb/metallb/v0.14.3/config/manifests/metallb-native.yaml |
| □ | 50 | Wait for MetalLB pods to be Running | kubectl wait --namespace metallb-system --for=condition=ready pod --selector=app=metallb --timeout=90s |
| □ | 51 | Create MetalLB IP address pool (save as metallb-pool.yaml and apply) | apiVersion: metallb.io/v1beta1 / kind: IPAddressPool / addresses: [192.168.10.200-192.168.10.220] |
| □ | 52 | Create MetalLB L2Advertisement (save as metallb-l2.yaml and apply) | apiVersion: metallb.io/v1beta1 / kind: L2Advertisement / namespace: metallb-system |
| □ | 53 | Verify MetalLB is ready | kubectl get ipaddresspool -n metallb-system |

PHASE H — llama-server DaemonSet (Kubernetes)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 54 | Create llm namespace | kubectl create namespace llm |
| □ | 55 | Create llama-config ConfigMap (save as llama-config.yaml and apply) | kind: ConfigMap / data: MODEL_PATH, N_GPU_LAYERS: 35, CONTEXT_SIZE: 4096, PARALLEL: 4, PORT: 8080 |
| □ | 56 | Create llama-server DaemonSet (save as llama-daemonset.yaml and apply) | kind: DaemonSet / image: ubuntu:22.04 / command: llama-server / resources.limits: nvidia.com/gpu: 1 / volumeMounts: /opt/llama.cpp (hostPath) |
| □ | 57 | Watch DaemonSet pods come up (one per node) | kubectl get pods -n llm -o wide -w # both pods STATUS=Running |
| □ | 58 | Check pod logs for GPU offload confirmation | kubectl logs -n llm -l app=llama-server # look for 'offloaded 35/35 layers to GPU' |
| □ | 59 | Test health endpoint via pod IP | kubectl get pods -n llm -o wide # get pod IP, then: curl <pod-ip>:8080/health |
| □ | 60 | Verify GPU usage on both nodes | nvidia-smi # VRAM ~4 GB used on each node |

PHASE I — nginx Ingress & Single Cluster Endpoint

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 61 | Deploy nginx Ingress Controller (bare-metal) | kubectl apply -f https://raw.githubusercontent.com/kubernetes/ingress-nginx/controller-v1.9.6/deploy/static/provider/baremetal/deploy.yaml |
| □ | 62 | Wait for ingress controller pod to be Ready | kubectl wait --namespace ingress-nginx --for=condition=ready pod --selector=app.kubernetes.io/component=controller --timeout=120s |
| □ | 63 | Patch ingress controller to use MetalLB LoadBalancer | kubectl patch svc ingress-nginx-controller -n ingress-nginx -p '{"spec":{"type":"LoadBalancer"}}' |
| □ | 64 | Get assigned external IP (should be 192.168.10.200) | kubectl get svc -n ingress-nginx ingress-nginx-controller # EXTERNAL-IP: 192.168.10.200 |
| □ | 65 | Create llama-service (ClusterIP, save as llama-service.yaml and apply) | kind: Service / selector: app=llama-server / port: 8080 / type: ClusterIP |
| □ | 66 | Create llama-ingress (save as llama-ingress.yaml and apply) | kind: Ingress / ingressClassName: nginx / path: / -> llama-service:8080 / proxy-read-timeout: 300 |
| □ | 67 | Test the single cluster endpoint | curl http://192.168.10.200/v1/chat/completions -H 'Content-Type: application/json' -d '{"model":"llama","messages":[{"role":"user","content":"Hello!"}]}' |
| □ | 68 | Send 10 requests and verify both pods serve traffic | for i in $(seq 1 10); do curl -s http://192.168.10.200/health; done |
| □ | 69 | Failover test: delete one pod, verify cluster still responds | kubectl delete pod -n llm <pod-name> && curl http://192.168.10.200/health # still ok |
| □ | 70 | Verify deleted pod auto-restarts | kubectl get pods -n llm -w # pod restarts automatically |
| □ | 71 | Reboot both nodes and confirm auto-start | sudo reboot # wait, then: curl http://192.168.10.200/health |

PHASE J — Validate & Tune the Cluster

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 72 | Run 5 quality test prompts via cluster endpoint | curl http://192.168.10.200/v1/chat/completions with factual questions |
| □ | 73 | Measure tokens/sec on a long generation | ./build/bin/llama-cli -m models/*.gguf -ngl 35 -p 'Write a 500 word essay' -n 500 2>&1 ¦ grep 'eval time' |
| □ | 74 | Tune -ngl: try 30, 33, 35 and compare speed | Lower -ngl = more CPU, less VRAM. 35 is optimal for 8B Q4_K_M on T1000. |
| □ | 75 | Tune context size: test -c 2048, 4096, 8192 | Larger context uses more RAM. 4096 is safe default with 64 GB RAM. |
| □ | 76 | Tune --parallel: test 2, 4, 8 concurrent slots | Higher parallel = more RAM used. 4 is safe for 64 GB. |
| □ | 77 | Update ConfigMap with final tuned values | kubectl patch configmap llama-config -n llm --type merge -p '{"data":{"N_GPU_LAYERS":"35","CONTEXT_SIZE":"4096","PARALLEL":"4"}}' |
| □ | 78 | Rolling restart DaemonSet to apply ConfigMap changes | kubectl rollout restart daemonset/llama-server -n llm |
| □ | 79 | Monitor rollout progress | kubectl rollout status daemonset/llama-server -n llm |
| □ | 80 | Set a custom system prompt for your use case in ConfigMap | Add SYSTEM_PROMPT key to llama-config ConfigMap, then rolling restart |
| □ | 81 | Document final working configuration | Save final ConfigMap YAML to /opt/cluster-config/llama-config.yaml |

PHASE K — Fine-Tune on Your Own Data (node-00)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 82 | Install Python fine-tuning dependencies on node-00 | pip install torch transformers datasets peft bitsandbytes trl accelerate |
| □ | 83 | Prepare training data in JSONL format | Each line: {"instruction": "...", "output": "..."} — min 100 examples recommended |
| □ | 84 | Write QLoRA fine-tuning script (finetune.py) | AutoModelForCausalLM(load_in_4bit=True) + LoraConfig(r=16, lora_alpha=32) + SFTTrainer |
| □ | 85 | Run fine-tuning on node-00 | python finetune.py # uses GPU + 64 GB RAM; expect 1-4 hours for 1000 examples |
| □ | 86 | Monitor GPU and RAM usage during training | watch -n 2 nvidia-smi # VRAM ~3.5-4 GB; htop for RAM |
| □ | 87 | Wait for training to complete (loss should decrease steadily) | Check output/trainer_state.json for final loss value |
| □ | 88 | Merge LoRA adapters into base model | python merge.py # AutoPeftModelForCausalLM.from_pretrained('./output').merge_and_unload() |
| □ | 89 | Install GGUF conversion requirements | pip install -r /opt/llama.cpp/requirements/requirements-convert_hf_to_gguf.txt |
| □ | 90 | Convert merged model to GGUF Q4_K_M | python /opt/llama.cpp/convert_hf_to_gguf.py ./merged_model --outtype q4_k_m --outfile finetuned-Q4_K_M.gguf |
| □ | 91 | Test fine-tuned model locally | ./build/bin/llama-cli -m finetuned-Q4_K_M.gguf -ngl 35 -p 'Test prompt from your domain' -n 128 |
| □ | 92 | Copy fine-tuned model to all nodes | rsync -avz finetuned-Q4_K_M.gguf ubuntu@node-01:/opt/llama.cpp/models/ |
| □ | 93 | Update ConfigMap MODEL_PATH to fine-tuned model | kubectl patch configmap llama-config -n llm --type merge -p '{"data":{"MODEL_PATH":"/opt/llama.cpp/models/finetuned-Q4_K_M.gguf"}}' |
| □ | 94 | Rolling restart to deploy fine-tuned model (zero downtime) | kubectl rollout restart daemonset/llama-server -n llm |
| □ | 95 | Monitor rollout | kubectl rollout status daemonset/llama-server -n llm |
| □ | 96 | Validate fine-tuned model via cluster endpoint | curl http://192.168.10.200/v1/chat/completions with domain-specific prompt |
| □ | 97 | Rollback if needed | kubectl rollout undo daemonset/llama-server -n llm |

PHASE L — Add a New Node (repeat per additional node)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 98 | Install Ubuntu Server 22.04 on new node | hostname: node-0N · IP: 192.168.10.1N · SSH enabled |
| □ | 99 | Add new node to /etc/hosts on all existing nodes | echo '192.168.10.1N node-0N' ¦ sudo tee -a /etc/hosts (on all nodes) |
| □ | 100 | Copy SSH key from node-00 to new node | ssh-copy-id ubuntu@node-0N |
| □ | 101 | Install NVIDIA driver 535 + CUDA 12 + nvidia-container-toolkit on new node | (same as Phase C steps 14-22) |
| □ | 102 | Build llama.cpp with CUDA on new node | (same as Phase D steps 23-28) |
| □ | 103 | rsync model files from node-00 to new node | rsync -avz /opt/llama.cpp/models/ ubuntu@node-0N:/opt/llama.cpp/models/ |
| □ | 104 | Configure nvidia-ctk for containerd on new node | sudo nvidia-ctk runtime configure --runtime=containerd |
| □ | 105 | Join new node to k3s cluster | curl -sfL https://get.k3s.io ¦ K3S_URL=https://192.168.10.10:6443 K3S_TOKEN=<TOKEN> sh - |
| □ | 106 | Verify new node is Ready | kubectl get nodes # new node STATUS=Ready |
| □ | 107 | DaemonSet auto-schedules llama pod on new node | kubectl get pods -n llm -o wide # new pod appears automatically |
| □ | 108 | Verify new node receives traffic | kubectl logs -n llm <new-pod-name> # should show inference requests |

PHASE M — Basic Monitoring (recommended)

| ✓ | # | Task | Command / Notes |
| --- | --- | --- | --- |
| □ | 109 | Install htop and nvtop on all nodes | sudo apt install -y htop nvtop |
| □ | 110 | Monitor GPU usage in real time | nvtop # GPU utilisation, VRAM, temperature per node |
| □ | 111 | Check DaemonSet pod logs | kubectl logs -n llm -l app=llama-server --tail=50 |
| □ | 112 | Check pod resource usage | kubectl top pods -n llm # requires metrics-server |
| □ | 113 | Install metrics-server for kubectl top | kubectl apply -f https://github.com/kubernetes-sigs/metrics-server/releases/latest/download/components.yaml |
| □ | 114 | Set up cron health check on node-00 (optional) | */5 * * * * curl -sf http://192.168.10.200/health ¦¦ kubectl rollout restart daemonset/llama-server -n llm |
| □ | 115 | Deploy Prometheus + Grafana when cluster is stable (optional) | helm install kube-prom prometheus-community/kube-prometheus-stack |
| □ | 116 | Deploy NVIDIA DCGM Exporter for GPU metrics (optional) | kubectl apply -f https://raw.githubusercontent.com/NVIDIA/dcgm-exporter/main/deployment/dcgm-exporter.yaml |

Summary: What You Have After Each Phase

| After Phase | You Have | Est. Time |
| --- | --- | --- |
| A | Hardware connected, BIOS configured, USB drives ready | 30 min |
| B | Both nodes running Ubuntu, SSH access, static IPs | 2-3 hours |
| C | NVIDIA drivers + CUDA working on both nodes | 1-2 hours |
| D | llama.cpp compiled with CUDA on both nodes | 30-60 min |
| E | Model downloaded, GPU inference working locally | 1-2 hours (download) |
| F | k3s cluster running, GPU scheduling validated | 1-2 hours |
| G | MetalLB ready, external IPs available | 20 min |
| H | llama-server DaemonSet running, one GPU pod per node | 30-60 min |
| I | Single cluster endpoint at 192.168.10.200, load balanced, auto-failover | 30 min |
| J-K | Tuned, fine-tuned model serving your use case via Kubernetes | 1-3 days (training) |

Total time to a working 2-node Kubernetes LLM cluster: approximately 1-2 days.

Adding each additional node: approximately 30-45 minutes (once you have the process memorised).

Key advantage of Kubernetes from day one: adding node 3, 4, or 10 requires zero changes to the serving stack. The DaemonSet and Service handle everything automatically.