LLM CLUSTER

Foundation Setup Guide

Hardware · OS · Network · Dependencies

Lenovo P1 Gen3 · Ubuntu 22.04 LTS · 2 Nodes → 10  |  2026

Table of Contents

1.  Hardware Overview

2.  BIOS Configuration

3.  OS Installation

4.  OS Configuration

5.  Network Configuration

6.  Firewall Configuration

7.  SSH Configuration

8.  Dependencies — Base Packages

9.  Dependencies — NVIDIA Drivers & CUDA

10.  Dependencies — Container Runtime

11.  Validation & Verification

12.  Scaling: Adding a New Node

13.  Troubleshooting

1. Hardware Overview

Each node in the cluster is a Lenovo ThinkPad P1 Gen3. The hardware is identical across all nodes. node-00 is the only node with a display — all others are headless.

Per-Node Specification

| Component | Specification | Notes |
| --- | --- | --- |
| CPU | Intel Core i7 vPro 10th Gen | 6-core / 12-thread, up to 5.0 GHz boost |
| RAM | 64 GB DDR4 | Handles KV cache overflow and future fine-tuning workloads |
| GPU | NVIDIA Quadro T1000 Max-Q | 4 GB GDDR6 VRAM, 896 CUDA cores, CUDA 12 capable |
| Storage | 512 GB – 1 TB NVMe SSD | OS + k3s + container images + GGUF model files |
| Network | 1 GbE Intel I219-LM | Sufficient for inference; upgrade path via USB-C 10GbE adapter |
| Display | node-00 only | All other nodes are headless — no monitor required |
| Power | ~65 W under load | ~130 W total for 2-node MVP; ~650 W for 10 nodes |

Node Inventory

| Hostname | IP Address | Role | Display |
| --- | --- | --- | --- |
| node-00 | 192.168.10.10 | k3s control plane + worker, admin access | Yes |
| node-01 | 192.168.10.11 | k3s worker | Headless |
| node-02 | 192.168.10.12 | k3s worker (future) | Headless |
| node-03 | 192.168.10.13 | k3s worker (future) | Headless |
| node-04 to node-09 | 192.168.10.14–19 | k3s workers (future) | Headless |

Physical Hardware Required (2-Node MVP)

| Item | Qty | Purpose | Notes |
| --- | --- | --- | --- |
| Lenovo ThinkPad P1 Gen3 (64 GB, T1000) | 2 | Cluster nodes | Already available |
| 1 GbE managed switch (8-port min) | 1 | Node interconnect | TP-Link TL-SG108E ~€30 |
| Cat6 Ethernet cables (2 m) | 2 | Node to switch | One per node |
| USB boot drives (16 GB min) | 2 | Ubuntu Server install | One per node |
| Power strip (4-outlet) | 1 | Power distribution | Both nodes + switch + spare |

Scale Path

| Nodes | Switch Required | Power | Notes |
| --- | --- | --- | --- |
| 2–5 | 8-port 1GbE managed | ~325 W | Current MVP switch sufficient |
| 6–10 | 16-port 1GbE managed | ~650 W | Upgrade switch; same config |
| 10+ training | 10GbE switch + USB-C NICs | ~650 W | Only if training sync is bottleneck |

2. BIOS Configuration

The following BIOS settings must be configured on EVERY node before installing Ubuntu. Incorrect BIOS settings are the most common cause of NVIDIA driver failures.

IMPORTANT: Secure Boot MUST be disabled. The NVIDIA proprietary driver will not load if Secure Boot is enabled. This is the single most common setup failure.

How to Enter BIOS on Lenovo ThinkPad P1 Gen3

Power on or reboot the node.

Press F1 repeatedly during the Lenovo splash screen (or press Enter, then F1).

Navigate using arrow keys. Save and exit with F10.

Required BIOS Settings

| Setting | Location in BIOS | Required Value | Why |
| --- | --- | --- | --- |
| Secure Boot | Security → Secure Boot | Disabled | NVIDIA proprietary driver will not load with Secure Boot enabled |
| Boot Mode | Startup → UEFI/Legacy Boot | UEFI Only | Required for Ubuntu Server 22.04 |
| Boot Order | Startup → Boot Order | USB first, then NVMe | Needed for Ubuntu installation from USB |
| Virtualization (VT-d) | Security → Virtualization | Enabled | Required for container GPU passthrough |
| Hyperthreading | Config → CPU | Enabled | Maximises CPU performance for CPU-side LLM layers |
| Wake on LAN | Config → Network | Enabled (optional) | Allows remote power-on of headless nodes |

Verify Secure Boot is Disabled (after Ubuntu install)

mokutil --sb-state
# Expected output: SecureBoot disabled
# If 'SecureBoot enabled' — return to BIOS and disable before proceeding

3. OS Installation

Install Ubuntu Server 22.04 LTS on every node. The installation is identical on all nodes — only the hostname and IP address differ.

Download and Flash USB Drive

Download Ubuntu Server 22.04 LTS ISO from ubuntu.com/download/server

Flash to USB drive using balenaEtcher (Windows/Mac) or dd (Linux):

sudo dd if=ubuntu-22.04-live-server-amd64.iso of=/dev/sdX bs=4M status=progress && sync

Repeat for each node (or reuse the same USB drive sequentially).

Installation Settings

| Setting | node-00 value | node-01 value | Notes |
| --- | --- | --- | --- |
| Hostname | node-00 | node-01 | Use node-02, node-03… for future nodes |
| Username | ubuntu | ubuntu | Same username on all nodes simplifies SSH |
| Password | (your choice) | (your choice) | Use same password for simplicity |
| Static IP | 192.168.10.10/24 | 192.168.10.11/24 | Set during install or via Netplan after |
| Gateway | 192.168.10.1 | 192.168.10.1 | Your router IP |
| DNS | 8.8.8.8, 1.1.1.1 | 8.8.8.8, 1.1.1.1 | Google + Cloudflare |
| SSH server | Install OpenSSH | Install OpenSSH | Check the box during install |
| Storage | Use entire disk | Use entire disk | No LVM needed |
| GUI | None | None | Server install — no desktop |

Installation Steps

Boot node from USB (F12 for boot menu on Lenovo, or set USB first in BIOS).

Select 'Try or Install Ubuntu Server'.

Choose language: English.

Choose keyboard layout.

Select 'Ubuntu Server' (not minimised).

Configure network: set static IP as shown in the table above.

Leave proxy blank.

Use default Ubuntu mirror.

Storage: use entire disk, no LVM.

Set hostname, username, and password as shown in the table above.

Check 'Install OpenSSH server'.

Do NOT install any snaps — skip this screen.

Wait for installation to complete (~10–15 minutes).

Remove USB drive when prompted, press Enter to reboot.

Log in with your username and password.

4. OS Configuration

Run all commands in this section on EACH node individually unless stated otherwise. Start with node-00, then repeat on node-01.

4.1 System Update

sudo apt update
sudo apt upgrade -y
sudo apt autoremove -y

4.2 Set Hostname (if not set during install)

sudo hostnamectl set-hostname node-00
echo "node-00" | sudo tee /etc/hostname
sudo sed -i 's/^127\.0\.1\.1.*/127.0.1.1\tnode-00/' /etc/hosts
hostname
# Expected: node-00

4.3 Set Timezone

sudo timedatectl set-timezone Europe/Berlin
timedatectl status

4.4 Firmware Update

sudo apt install -y fwupd
sudo fwupdmgr refresh --force
sudo fwupdmgr get-updates
# Review available updates — do NOT auto-apply
# To apply: sudo fwupdmgr update

4.5 Install Base Packages

sudo apt install -y \
    curl wget git vim htop nvtop \
    build-essential cmake unzip rsync \
    net-tools dnsutils ufw \
    ca-certificates gnupg lsb-release \
    openssh-server python3 python3-pip

4.6 Disable Swap (required for Kubernetes)

sudo swapoff -a
sudo sed -i '/\sswap\s/s/^/#/' /etc/fstab
free -h
# Expected: Swap line shows 0B

4.7 Load Required Kernel Modules

These kernel modules are required for k3s networking (Flannel VXLAN) and Kubernetes network policies.

sudo modprobe overlay
sudo modprobe br_netfilter

sudo tee /etc/modules-load.d/k3s.conf << 'EOF'
overlay
br_netfilter
EOF

sudo tee /etc/sysctl.d/99-k3s.conf << 'EOF'
net.bridge.bridge-nf-call-iptables  = 1
net.bridge.bridge-nf-call-ip6tables = 1
net.ipv4.ip_forward                 = 1
EOF

sudo sysctl --system

5. Network Configuration

All nodes use static IP addresses configured via Netplan. Static IPs are essential — Kubernetes and k3s require stable node addresses. DHCP must not be used.

5.1 Identify Network Interface

ip link show
# Look for the interface that is NOT 'lo' (loopback)
# Common names on P1 Gen3: enp0s31f6, eno1, eth0
# Note your interface name — replace 'enp0s31f6' in the config below

5.2 Netplan Static IP Configuration

Create or replace the Netplan config file. Use the correct IP address for each node. Replace enp0s31f6 with your actual interface name.

# Backup existing config first
sudo cp /etc/netplan/*.yaml /etc/netplan/backup-$(date +%Y%m%d).yaml 2>/dev/null || true

# Write new config
# IMPORTANT: replace enp0s31f6 with your interface name
# IMPORTANT: replace 192.168.10.10 with the correct IP for this node
sudo tee /etc/netplan/01-cluster.yaml << 'EOF'
network:
  version: 2
  renderer: networkd
  ethernets:
    enp0s31f6:
      dhcp4: false
      addresses:
        - 192.168.10.10/24
      routes:
        - to: default
          via: 192.168.10.1
      nameservers:
        addresses:
          - 8.8.8.8
          - 1.1.1.1
EOF

sudo chmod 600 /etc/netplan/01-cluster.yaml
sudo netplan apply

# Verify
ip addr show enp0s31f6
# Expected: inet 192.168.10.10/24
ping -c 3 8.8.8.8

5.3 IP Address Reference

| Hostname | IP Address | Subnet | Role |
| --- | --- | --- | --- |
| node-00 | 192.168.10.10 | /24 | k3s control plane + worker |
| node-01 | 192.168.10.11 | /24 | k3s worker |
| node-02 | 192.168.10.12 | /24 | k3s worker (future) |
| node-03 | 192.168.10.13 | /24 | k3s worker (future) |
| node-04 | 192.168.10.14 | /24 | k3s worker (future) |
| node-05 | 192.168.10.15 | /24 | k3s worker (future) |
| node-06 | 192.168.10.16 | /24 | k3s worker (future) |
| node-07 | 192.168.10.17 | /24 | k3s worker (future) |
| node-08 | 192.168.10.18 | /24 | k3s worker (future) |
| node-09 | 192.168.10.19 | /24 | k3s worker (future) |
| MetalLB pool | 192.168.10.200–220 | /24 | Reserved for Kubernetes LoadBalancer services |
| Gateway | 192.168.10.1 | /24 | Router |

5.4 /etc/hosts — Cluster Name Resolution

Add all cluster node hostnames to /etc/hosts on EVERY node. This allows nodes to reach each other by hostname without requiring a DNS server.

sudo tee -a /etc/hosts << 'EOF'

# LLM Cluster nodes
192.168.10.10   node-00
192.168.10.11   node-01
192.168.10.12   node-02
192.168.10.13   node-03
192.168.10.14   node-04
192.168.10.15   node-05
192.168.10.16   node-06
192.168.10.17   node-07
192.168.10.18   node-08
192.168.10.19   node-09
EOF

# Verify from node-00:
ping -c 2 node-01
# Expected: replies from 192.168.10.11

5.5 Subnet Layout

192.168.10.0/24

  .1    Router / Gateway
  .10   node-00  ── k3s control plane + worker  [screen]
  .11   node-01  ── k3s worker                  [headless]
  .12   node-02  ── k3s worker (future)         [headless]
  ...   ...
  .19   node-09  ── k3s worker (future)         [headless]

  .200–.220   MetalLB pool
              .200 = nginx Ingress (cluster API endpoint)

All nodes ── 1GbE managed switch ── router
Scale: upgrade to 16-port switch at 6+ nodes
       upgrade to 10GbE when training sync is bottleneck

6. Firewall Configuration

UFW is configured on all nodes to allow all required cluster traffic from the internal subnet while blocking everything else. Run on EVERY node.

6.1 UFW Rules

sudo ufw allow OpenSSH
sudo ufw allow from 192.168.10.0/24 to any port 22    comment 'SSH cluster'
sudo ufw allow from 192.168.10.0/24 to any port 6443  comment 'k3s API server'
sudo ufw allow from 192.168.10.0/24 to any port 10250 comment 'k3s kubelet'
sudo ufw allow from 192.168.10.0/24 to any port 10255 comment 'k3s kubelet read-only'
sudo ufw allow from 192.168.10.0/24 to any port 8472  comment 'Flannel VXLAN'
sudo ufw allow from 192.168.10.0/24 to any port 51820 comment 'Flannel WireGuard'
sudo ufw allow from 192.168.10.0/24 to any port 2379  comment 'etcd client'
sudo ufw allow from 192.168.10.0/24 to any port 2380  comment 'etcd peer'
sudo ufw allow from 192.168.10.0/24 to any port 8080  comment 'llama-server'
sudo ufw allow from 192.168.10.0/24 to any port 8090  comment 'pipeline API'
sudo ufw allow from 192.168.10.0/24 to any port 3000  comment 'Grafana'
sudo ufw allow from 192.168.10.0/24 to any port 9090  comment 'Prometheus'
sudo ufw --force enable
sudo ufw status numbered

6.2 Port Reference

| Port | Protocol | Service | Required on |
| --- | --- | --- | --- |
| 22 | TCP | SSH | All nodes |
| 6443 | TCP | k3s API server | node-00 (control plane) |
| 10250 | TCP | kubelet API | All nodes |
| 10255 | TCP | kubelet read-only | All nodes |
| 8472 | UDP | Flannel VXLAN overlay | All nodes |
| 51820 | UDP | Flannel WireGuard (if enabled) | All nodes |
| 2379–2380 | TCP | etcd (k3s embedded) | node-00 only |
| 8080 | TCP | llama-server inference | All nodes |
| 80 / 443 | TCP | nginx Ingress | node-00 (MetalLB .200) |
| 9090 | TCP | Prometheus | node-00 |
| 3000 | TCP | Grafana | node-00 |

7. SSH Configuration

Passwordless SSH from node-00 to all worker nodes is required for cluster management, Ansible automation, and model file synchronisation via rsync.

7.1 Generate SSH Key on node-00

ssh-keygen -t ed25519 -f ~/.ssh/id_ed25519 -N ''
cat ~/.ssh/id_ed25519.pub
# Copy this output — you will need it for each worker node

7.2 Copy Key to Worker Nodes

ssh-copy-id ubuntu@node-01
ssh-copy-id ubuntu@node-02   # when added

# Test passwordless SSH
ssh node-01 hostname
# Expected: node-01  (no password prompt)

7.3 Fix SSH Permissions (if passwordless SSH fails)

chmod 700 ~/.ssh
chmod 600 ~/.ssh/authorized_keys
chmod 600 ~/.ssh/id_ed25519
chmod 644 ~/.ssh/id_ed25519.pub

sudo sed -i 's/^#*PubkeyAuthentication.*/PubkeyAuthentication yes/' /etc/ssh/sshd_config
sudo systemctl restart ssh

7.4 sshd_config Hardening (recommended)

sudo tee /etc/ssh/sshd_config.d/cluster.conf << 'EOF'
PubkeyAuthentication yes
AuthorizedKeysFile .ssh/authorized_keys
PasswordAuthentication yes
PermitRootLogin no
X11Forwarding no
EOF
sudo systemctl restart ssh

8. Dependencies — Base Packages

Install on EVERY node. These packages are required before NVIDIA drivers, k3s, and llama.cpp can be installed.

8.1 Core Build Tools

sudo apt install -y \
    build-essential cmake git make gcc g++ pkg-config

gcc --version
cmake --version
git --version

8.2 System Utilities

sudo apt install -y \
    curl wget unzip rsync htop nvtop \
    net-tools dnsutils lsof jq vim tmux

8.3 Python

Required for llama.cpp model conversion scripts and future fine-tuning workloads.

sudo apt install -y python3 python3-pip python3-venv python3-dev
python3 --version
pip3 install --upgrade pip

8.4 Container Prerequisites

sudo apt install -y \
    ca-certificates gnupg lsb-release apt-transport-https

8.5 Package Summary

| Package Group | Packages | Purpose |
| --- | --- | --- |
| Build tools | build-essential, cmake, git, gcc, g++ | Compile llama.cpp with CUDA support |
| Utilities | curl, wget, unzip, rsync, htop, nvtop, jq, tmux | System management and file transfer |
| Python | python3, python3-pip, python3-venv, python3-dev | Model conversion and fine-tuning |
| Container prep | ca-certificates, gnupg, lsb-release, apt-transport-https | Required before adding NVIDIA and Docker repos |
| Network | net-tools, dnsutils, lsof | Network diagnostics |

9. Dependencies — NVIDIA Drivers & CUDA

Install on EVERY node. SECURE BOOT MUST BE DISABLED IN BIOS BEFORE THIS STEP.

CRITICAL: Run 'mokutil --sb-state' before proceeding. If it shows 'SecureBoot enabled', stop and disable Secure Boot in BIOS first. This is the most common cause of nvidia-smi failures.

9.1 Pre-flight Check

mokutil --sb-state
# Expected: SecureBoot disabled

lsmod | grep nouveau
# If output is not empty, blacklist it:
sudo bash -c 'cat > /etc/modprobe.d/blacklist-nouveau.conf << EOF
blacklist nouveau
options nouveau modeset=0
EOF'
sudo update-initramfs -u -k all
# Then reboot before continuing

9.2 Add NVIDIA CUDA Repository

wget https://developer.download.nvidia.com/compute/cuda/repos/ubuntu2204/x86_64/cuda-keyring_1.1-1_all.deb
sudo dpkg -i cuda-keyring_1.1-1_all.deb
sudo apt update

9.3 Install NVIDIA Driver 535

sudo apt install -y nvidia-driver-535

sudo tee /etc/apt/preferences.d/nvidia-pin << 'EOF'
Package: nvidia-driver-*
Pin: version 535.*
Pin-Priority: 1001
EOF

9.4 Install CUDA Toolkit 12.3

sudo apt install -y cuda-toolkit-12-3

sudo tee -a /etc/apt/preferences.d/nvidia-pin << 'EOF'
Package: cuda-toolkit-*
Pin: version 12.3.*
Pin-Priority: 1001
EOF

sudo tee /etc/profile.d/cuda.sh << 'EOF'
export PATH=/usr/local/cuda/bin:$PATH
export LD_LIBRARY_PATH=/usr/local/cuda/lib64:$LD_LIBRARY_PATH
EOF

9.5 Reboot and Validate

sudo reboot

# After reboot:
nvidia-smi
# Expected:
# +-----------------------------------------------------------------------------+
# | NVIDIA-SMI 535.x    Driver Version: 535.x    CUDA Version: 12.3            |
# | GPU 0: Quadro T1000 Max-Q   4096 MiB total   0 MiB used                   |
# +-----------------------------------------------------------------------------+

nvcc --version
# Expected: Cuda compilation tools, release 12.3

9.6 Confirmed Hardware Constraints

These values were measured on the actual hardware and must be used in all llama.cpp and Kubernetes configurations:

| Parameter | Value | Notes |
| --- | --- | --- |
| VRAM available | 4 GB (4096 MiB) | T1000 Max-Q hard limit |
| Working -ngl value | 20 | Q4_K_M model weights (4.7 GB) exceed 4 GB VRAM; 20 layers fit on GPU |
| Remaining layers | 15 (CPU) | Run on 64 GB RAM — comfortable, no OOM risk |
| Context window | 6144 tokens | Tested and working at -ngl 20 |
| Inference speed | ~12 t/s per node | At -ngl 20 with Q4_K_M |
| Q3_K_M alternative | ~20 t/s | Fits all 35 layers on GPU; test quality before committing |

10. Dependencies — Container Runtime

k3s uses containerd as its container runtime. The nvidia-container-toolkit must be installed and configured for containerd BEFORE k3s is installed, so that GPU passthrough works from day one.

10.1 Install nvidia-container-toolkit

sudo apt install -y nvidia-container-toolkit
nvidia-container-cli --version

10.2 Configure containerd for NVIDIA

sudo nvidia-ctk runtime configure --runtime=containerd

sudo grep -A5 'nvidia' /etc/containerd/config.toml 2>/dev/null || \
  echo 'Config will be applied when k3s installs containerd'

10.3 Install Docker (optional)

curl -fsSL https://get.docker.com | sh
sudo usermod -aG docker $USER

sudo nvidia-ctk runtime configure --runtime=docker
sudo systemctl restart docker

docker run --rm --gpus all nvidia/cuda:12.3.0-base-ubuntu22.04 nvidia-smi
# Expected: same nvidia-smi output as bare metal

10.4 Re-configure containerd after k3s install

k3s installs its own containerd instance at a different path. After k3s is installed, run this again to ensure GPU passthrough works inside k3s pods.

sudo nvidia-ctk runtime configure --runtime=containerd
sudo systemctl restart k3s          # node-00
sudo systemctl restart k3s-agent    # node-01+

grep -A5 'nvidia' /var/lib/rancher/k3s/agent/etc/containerd/config.toml

11. Validation & Verification

Run all checks on EVERY node before proceeding to cluster setup. Every item must pass. Do not proceed to k3s installation until all checks are green.

Full Validation Checklist

| Check | Command | Expected Result |
| --- | --- | --- |
| Hostname correct | hostname | node-00 (or node-01, etc.) |
| Static IP set | ip addr show ¦ grep 'inet ' | 192.168.10.1X/24 |
| Gateway reachable | ping -c 3 192.168.10.1 | 3 packets received, 0% loss |
| Internet reachable | ping -c 3 8.8.8.8 | 3 packets received, 0% loss |
| DNS working | nslookup google.com | Non-authoritative answer |
| Peer node reachable | ping -c 3 node-01 | 3 packets received (from node-00) |
| SSH passwordless | ssh node-01 hostname | node-01 (no password prompt) |
| Secure Boot off | mokutil --sb-state | SecureBoot disabled |
| NVIDIA driver | nvidia-smi | T1000, 4096 MiB, Driver 535.x |
| CUDA toolkit | nvcc --version | release 12.3 |
| GPU in container | docker run --rm --gpus all nvidia/cuda:12.3.0-base-ubuntu22.04 nvidia-smi | T1000 visible in container |
| Swap disabled | free -h | Swap: 0B |
| Kernel modules | lsmod ¦ grep -E 'overlay¦br_netfilter' | Both modules listed |
| UFW active | sudo ufw status | Status: active, rules listed |
| Python available | python3 --version | Python 3.10.x or 3.11.x |

Quick Validation Script

Save as /opt/validate.sh on each node and run: bash /opt/validate.sh

#!/bin/bash
echo '=== Node Validation: '$(hostname)' ==='
echo -n 'Hostname:        '; hostname
echo -n 'IP address:      '; hostname -I | awk '{print $1}'
echo -n 'Secure Boot:     '; mokutil --sb-state 2>/dev/null || echo 'mokutil not installed'
echo -n 'NVIDIA driver:   '; nvidia-smi --query-gpu=driver_version --format=csv,noheader 2>/dev/null || echo 'FAILED'
echo -n 'CUDA version:    '; nvcc --version 2>/dev/null | grep release | awk '{print $6}' || echo 'FAILED'
echo -n 'GPU VRAM:        '; nvidia-smi --query-gpu=memory.total --format=csv,noheader 2>/dev/null || echo 'FAILED'
echo -n 'Swap:            '; free -h | awk '/Swap/{print $2}'
echo -n 'UFW status:      '; sudo ufw status | head -1
echo -n 'Python:          '; python3 --version
echo -n 'Kernel modules:  '; lsmod | grep -cE 'overlay|br_netfilter' | xargs -I{} echo '{}/2 loaded'
echo '=== Done ==='

12. Scaling: Adding a New Node

Every new node follows the exact same process as node-01. The only values that change are the hostname and IP address. Estimated time per node: 30–45 minutes once the process is familiar.

New Node Checklist

Set hostname: node-0N (where N = 2, 3, 4, …)

Set static IP: 192.168.10.1N in Netplan.

Add new node's IP + hostname to /etc/hosts on ALL existing nodes.

Add all existing node IPs + hostnames to /etc/hosts on the new node.

Run Section 4 (OS Configuration) on new node.

Run Section 5 (Network Configuration) on new node.

Run Section 6 (Firewall) on new node.

Run Section 7 (SSH) — copy node-00's public key to new node.

Run Section 8 (Base Packages) on new node.

Run Section 9 (NVIDIA Drivers & CUDA) on new node.

Run Section 10 (Container Runtime) on new node.

Run Section 11 (Validation) on new node — all checks must pass.

Proceed to k3s cluster setup to join the new node (covered in the Cluster Configuration guide).

Switch Upgrade (at 6+ nodes)

Replace the 8-port switch with a 16-port managed switch. No configuration changes are needed — just reconnect the cables. All static IPs remain the same.

Netplan IP Reference for New Nodes

# node-02
addresses: [192.168.10.12/24]

# node-03
addresses: [192.168.10.13/24]

# node-04
addresses: [192.168.10.14/24]

# ... up to node-09 at 192.168.10.19/24

13. Troubleshooting

nvidia-smi Failures

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| 'No devices were found' | Secure Boot enabled | Disable in BIOS → reboot |
| 'NVIDIA-SMI has failed' | Driver not loaded after install | sudo modprobe nvidia; if fails → reboot |
| nvidia-smi: command not found | Driver not installed | Re-run Section 9 |
| Wrong driver version shown | Version pinning not applied | sudo apt install -y nvidia-driver-535 |
| nouveau still loaded | Blacklist not applied | Run blacklist commands → sudo update-initramfs -u -k all → reboot |
| Driver loads but GPU not visible | VT-d disabled in BIOS | Enable Virtualization in BIOS → reboot |

Network & SSH Issues

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Cannot ping node-01 from node-00 | /etc/hosts missing entry | echo '192.168.10.11 node-01' ¦ sudo tee -a /etc/hosts |
| SSH asks for password | Wrong key permissions or key not copied | chmod 700 ~/.ssh && chmod 600 ~/.ssh/authorized_keys; re-run ssh-copy-id |
| SSH connection refused | sshd not running | sudo systemctl enable --now ssh |
| Static IP not applied | Netplan not applied or YAML error | sudo netplan generate (check for errors); sudo netplan apply |
| IP reverts to DHCP after reboot | Old DHCP config still active | Remove old netplan files; keep only 01-cluster.yaml |
| Cannot reach internet | Gateway wrong or DNS missing | Check Netplan routes and nameservers; ping 192.168.10.1 first |

Netplan YAML Syntax Check

sudo netplan generate
# If no output: syntax is correct
# If errors: fix YAML indentation (use spaces, NOT tabs)

General Diagnostics

systemctl status ssh
systemctl status ufw
nvidia-smi
nvcc --version
ip addr show
ip route show
ping -c 3 192.168.10.1
lsmod | grep -E 'overlay|br_netfilter|nvidia'
free -h
cat /etc/hosts